INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11/N18501

March 2019, Geneve, CH

Title:		White paper on Media Orchestration (MORE)

Source		Communication

Status:	Approved

N18501N18501

Media Orchestration between Streams and Devices via new MPEG Timed Metadata, MPEG-MORE (Media ORchEstration)

Introduction

A typical household may own more than 10 internet-connected media devices, including smart TVs, tablet devices, smartphones and smart watches. The combined use of devices may enhance the media consumption experience and new opportunities for media orchestration also arise at the capture side, as the number of cameras, microphones while sensors (location, orientation) may match the number of people present at large sports, music or other events. Moreover, the emergence of 360o video (“virtual reality”) and associated 3D audio offer opportunities for less TV-centric orchestrations of media capture, processing and rendering.

However, when consuming multiple types of media at multiple sources and sinks, each media normally has different (types of) timelines and spatial locations. For instance, a source or sink device cannot consume both an ISOBMFF file which has composed time stamps and static Cartesian coordinate information and an MPEG-DASH stream with media presentation timestamps and dynamic GPS locations. Therefore, we need a description of the correlation or translation between the different types of timelines, as well as a standardized way to describe static and dynamic spatial relationships.

MPEG (Moving Picture Experts Group) initiated its Media Orchestration (“MORE”) activity early 2015, to create tools to achieve such media orchestration. Media Orchestration allows coordination of sources and sinks in time and space.

- In time, media orchestration allows devices and apps (“sinks”) to consume multiple media streams from diverse sources with different types of media timelines by using correlation timestamps

- In space, the MORE specification defines static, but more importantly, dynamic metadata for describing spatial information associated with the media.

This paper discusses various aspects of media orchestration, including use cases, details of the functional architecture, and which metadata is defined – along with the ways to carry it.

Needs of media orchestration

MPEG builds its standards on a set of requirements, which are typically derived from a set of use cases. MPEG collected a large number of use cases that require the orchestration of media devices, and then combined these to have a set of distinct use cases that were clearly different, requiring clearly different functionality. The use cases below are adapted from these.

Advanced multi-camera video stitching

The demand for wide-field-of-view video is increasing to provide immersiveness. It requires panoramic videos with wide horizontal and vertical angles. Figure 1 presents a simple comparison between several video formats. Even though it depends on shooting environments, multiple camera systems are usually able to provide wide viewing angles which are enough to cover the whole human vision area. For that purpose, there are several specific solutions to keep the pixel density and a distortion-free view.

Figure 1 – Immersive experience from wide view angle video

Figure 2 shows two types of multiple camera systems and a real-time monitoring system. The use of multiple cameras reduces radial distortion caused by wide-viewing-angle lenses. However, it requires a stitching process supported by high-performance GPUs to produce seamless video with multiple video streams in real-time. This complicated process requires media orchestration in the form of spatial information of multiple cameras and target objects, as well as temporally synchronized video streams.

Figure 2 – Multiple camera systems and real-time monitoring system for panoramic video

Figure 3 – Tracking people over multiple street views

Tracking persons of interest over multiple street views

In the security domain, investigators are required to construct an event narrative, by stitching together a single video stream obtained from multiple cameras of different types, see Figure 3. The orchestration methodology for combining CCTV footage with different types of user content requires spatial and temporal, media orchestration based on Distinctive Regions or Patterns (DROP) [3]. In DROP-based media orchestration, each captured video stream has its own timeline, different and independent of the other video streams. The use cases require correlation of timelines between the different video streams. The media orchestration specification provides architectural components and data formats to enable the provision of universal media timeline in which heterogeneous timed media can be represented from disparate sources.

Virtual reality related media orchestration use case

Multiple resources are used to generate virtual reality and augmented reality contents. Multiple cameras are required to capture 360 degree and in addition to the cameras, graphical contents are required to generate augmented reality contents. The Media Orchestration specification will provide technologies to efficiently implement such use cases.

One of the interesting use cases along this line is generating content covering 360 degrees with large number of uncoordinated video feeds. On the location of a concert/festival, there are usually a number of video feeds, both professional (good quality, reliable, continuous) and amateur (any quality, unreliable, on and off randomly), as well as picture contributions (of all qualities) to the capture scene. A (e.g. distant) user will have a means to browse the capture scene and select a point of view dynamically. The system implementing such use case requires to have means to dynamically orchestrate the media resources and stitch input video feeds according to the view selected by a user. In some cases, a video corresponding to a synthetic point of view only partially covered by some video feeds, in which case a combination of photos and videos dynamically selected are used to construct the synthetic point of view, will be generated.

Immersive Coverage of Spatially Outspread Live Events (ICoSOLE) is a project developing a system for such use case [4]. ICoSOLE aims at supporting use cases that enables users to experience live events which are spatially spread out, such as festivals (e.g. Gentse feesten in Belgium, Glastonbury in the UK), parades, marathons or bike races, in an immersive way by combining high-quality spatial video and audio and user generated content.

Architecture for media orchestration

Media orchestration is about capture and consumption of many media with the help of (timed) metadata. The scope of the specification includes any combination of media and metadata to produce more media and metadata: synchronisation, stitching/mixing, composition, and more. Capture and consumption can be mixed quite intricately in some applications, but conceptually they are separate.

Figure 4 – Architecture of Media Orchestration

Figure 4 illustrates the media orchestration architecture. On the capture side, there are “sources”: sources of media and/or sources of metadata. On the consumption side there are “sinks”, which present media to consumers according to metadata and orchestration information. On both sides, processors may transform media and/or metadata, and controllers create orchestration information and control the orchestration through messaging.

Relevant media types include audio, video, 3D, 360 video, etc. Relevant metadata types include anything from synchronization metadata (spatial, temporal) to semantic information, capture location and direction to object position tracking in a media.

In this architecture, there are four abstract types of functional elements:

- Source: A source captures media and/or creates metadata and is capable to stream them. A video camera is a source of media. A GPS is a source of location metadata. A source registers with a controller and then listens to its messages: such messages include to start or stop the capture, to configure capture parameters including encoding, format, quality, focal length, capture direction, etc.

- Sink: A sink presents one or more media to consumers, possibly driven by metadata such as composition metadata. Multiple media maybe presented on one sink, which is enabled by scene or composition metadata. Multiple sinks may be used to present a single media or a synchronized presentation with a help of a particular processor.

- Processor: A processor transforms media and/or metadata into media and/or metadata. A transcoder from a video format to another is a possible media-to-media processor. An analyser generating football player positions from a video stream is an example of a media-to-metadata processor. A video stitcher, transforming multiple videos into one 360 video, is another example of processor.

- Controller: A controller keeps a list of available sources, sinks and processors; and it organizes connections, distribution, storage and retrieval of data. One controller can manage both capture and consumption for some applications where both happen at the same time; or there can be a controller (or more) on each side, for example if consumption happens later.

In Figure 4, the capture controller registers the sources, connects them to the processor inputs and to distribution, triggers capture and processing and passes information to the consumption controller. The consumption controller registers the sinks and processors, connects processor outputs and distribution to sinks. Controllers manage new sources and sinks or their variable availability, locations, quality. The difference between a controller and a processor is that a controller receives and sends messages dealing with the setup of media orchestration, whereas a processor deals with data used for the consumer experience, either directly media, or data used to be able to present media, such as synchronization or composition information.

A media-orchestration application (outside of the scope of the standard) would run on top of the controllers. Such an application could enable editing by a human director, presenting all the information available to the controllers in a concise manner, and relaying directions. The MPEG MORE standard is planned to include APIs allowing the application to make use of web technologies for such a user interface.

Temporal orchestration

The MPEG MORE standard provides correlation timestamps for the temporal orchestration. Correlation stamps are a timed metadata stream that describes the correlation between two media timelines Because correlation timestamps are timed themselves, they can support media streams that have a “drift”. The formal definition is:

- Correlation timestamps: A data object that associates two or more timestamps of two or more timed data with each other. A typical example is the association between a presentation timestamp (PTS) of an MPEG-2 transport stream and a composition time of an ISOBMFF file.

Two DVB CSS protocols  for temporal orchestration are reused by MPEG MORE: the Wall Clock (WC) protocol and the Timeline Synchronisation (TS) protocol, see Figure 5.

- The WC protocol creates a uniform and consistent reference clock on all the synchronised devices. The term “wall clock” is a bit of a misnomer, as the wall clock time is unrelated to UTC or local time.

- The TS protocol coordinates the wall clock times at which identified video frames or audio samples should be presented to the user on the different devices. The devices each report the earliest wall-clock time that they could present an identified video frame or audio sample and coordinate playout delays accordingly. Video frames and audio samples are identified by timestamps in their media container, e.g. composition timestamp (ISO Based Media File Format) or presentation timestamp (MPEG-2 Transport Stream).

MPEG MORE extends various aspects of DVB CSS. Whereas DVB CSS focusses on timed media data, MPEG MORE also includes timed metadata. An example is the timeline synchronisation between a video stream and a stand-alone position stream, e.g. a timed stream of GPS data. As timed media data and timed metadata are carried in the same types of containers, DVB CSS can be reused as is for timed metadata.

Figure 5 – Architecture for temporal orchestration [1].

Whereas DVB CSS covers only render/sink-side timeline synchronisation, MPEG MORE also covers capture/source-side timeline synchronisation. The TS protocol is extended for this purpose, coordinating the wall clock time at which identified video frames or audio samples are captured.

A third aspect is timeline correlation. If different timed media or timed metadata have different time bases, then it is needed to know the clock skew, clock drift and clock-drift variation between the two time-bases. As the DVB CSS solution for this was too TV centric, MPEG MORE specifies its own solution, namely metadata for timeline correlation.

Spatial orchestration

MPEG MORE considers use cases with multiple sources as well as sinks including one-to-many and many-to-one scenarios. One or more streams from multiple sources are dynamically played out across multiple sinks. These multiple media streams can be combined to provide a single immersive experience, e.g. omnidirectional video.

In order to achieve this, MPEG MORE considers how to arrange and coordinate media streams based on its spatial relationships when the location and orientation (gaze) of sources are tracked in a 3D environment.

For example, when a tracker (that is able of tracking location and orientation) is attached to a camera, the location and orientation of the camera are continuously tracked as the video is being captured. With a captured video stream, the position and orientation streams of the camera are also generated. The position and orientation streams are timed metadata about the associated video stream according to its intrinsic timeline. Both media and metadata streams are delivered to the sinks, or they can be used by processors that exist between sources and sinks. For spatial orchestration of multiple media streams generated from independent sources, the position and orientation streams are used to recognize the spatial relationship of associated media streams and to coordinate the media streams. Metadata can also be used for supporting dynamic media presentation according to the sink’s movement.

These are the main timed metadata for the spatial relationship description.

- Position Metadata

- Orientation Metadata

- Region of Interest Metadata

- Capture Mask Metadata

- Stream Monitor Metadata

- Camera Metadata

- Depth Camera Metadata

Carriage of timed metadata for media orchestration

Media-orchestration metadata is defined as data that cannot be rendered independently and may affect rendering, processing or orchestration of the associated media data. Like media data, this metadata can be timed metadata which has an intrinsic timeline. For supporting temporal or spatial orchestration of multiple media streams, MPEG MORE describes several types of timed metadata, e.g., position, altitude, orientation, quality, stream monitor, and specifies how to carry the timed metadata in two existing MPEG file format.

- Carriage of timed metadata in ISO Base Media File Format (ISOBMFF)

- Carriage of timed metadata in MPEG-2 transport streams (TS).

The carriage of timed metadata in ISOBMFF is useful in cases where media data and associated timed metadata are stored in files, either together or in separate files. In this case, the timed metadata is carried in the metadata tracks within an ISOBMFF file. Different metadata types and corresponding storage formats are identified by their unique sample entry codes.

The carriage of timed metadata in MPEG-2 TS (Transport Stream) is useful in cases of broadcast of media data and associated timed metadata. The timed metadata associated with one or more video or audio frame are stored in access units and encapsulated in a PES stream.

Conclusions

This paper introduced the MPEG specification on media orchestration (MORE). This specification provides metadata and protocols for temporal and spatial orchestration between multiple media capture and rendering devices. Use cases include advanced multi-camera video stitching, tracking persons of interest over multiple street views as well as immersive media. The specification provides an architecture with sources, sinks, processors and controllers, as well as audio-visual media data, metadata and orchestration data. Temporal orchestration (synchronisation of media data and metadata) is achieved by reusing and extending protocols from DVB CSS. Spatial orchestration is achieved with new timed location and orientation metadata that is associated with the audio-visual media data. The new timed metadata is specified to be carried in ISOBMFF files as well as MPEG-2 Transport streams.

References

[1]	MPEG, “Media Orchestration” (MORE), ISO/IEC 23001-13:2018, draft available via authors

[2]	HbbTV, “Hybrid Broadcast Broadband Television version 2.0”, ETSI TS 102 796 V1.4.1, 2016, http://www.etsi.org/deliver/etsi_ts/102700_102799/102796

[3]	LASIE, “Large-scale information exploitation of forensic data”, http://www.lasie-project.eu

[4]	ICoSOLE, “Immersive Coverage of Spatially Outspread Live Events”, http://icosole.eu/public-deliverables.

[5]	DVB, “Companion Streams and Streams”, ETSI TS 103 286 02 v1.1.1, http://www.etsi.org/deliver/etsi_ts/103200_103299/10328602, 2015

[6]	M. Oskar van Deventer, Hans Stokking, Matt Hammond, Jean Le Feuvre, Pablo Cesar, “Standards for Multi-stream and Multi-device Media Synchronization”, IEEE Communications Magazine — Communications Standards Supplement, March 2016, pp.16-21p

ACKNOWLEDGEMENT

This paper was first presented at IBC2017.